Concordance file	Last edited 9/30/2026

On page 30, step 4a. in Chapter 1, the command to install Webmin is-

sudo apt-get install --install-recommends webmin usermin

1.2.6 Replacing the Existing NVMe SSD on a Raspberry Pi 500+ with a Larger Capacity Device

Objectives: To do a bare metal replacement of the 256 GB NVMe SSD that comes with a new Raspberry Pi 500+ with a larger capacity SSD, in our example here, a 1 TB NVMe SSD, and download and install the latest version of the OS with the use of the Raspberry Pi Imager. The version of the OS that comes pre-installed on the 256 GB NVMe SSD when you buy a new Raspberry Pi 500+ is probably not as current as the latest release of the OS.

Pre-Requisites:

a) We do the operations in this Section on the following Raspberry Pi 500+ system, which has the original 256 GB NVMe SSD with the default OS loaded on it, as follows:

System:

Host: raspberrypi Kernel: 6.6.74+rpt-rpi-2712 arch: aarch64 bits: 64

compiler: gcc v: 12.2.0 Desktop: wayfire v: 0.7.5 Distro: Debian GNU/Linux

12 (bookworm)

b) Having a USB3 NVMe adapter available that works on your Raspberry Pi 500+, such as our recommended Eluteng SATA-to-USB tool-less enclosure, to copy the latest version of the OS onto a new, higher-capacity NVMe SSD contained in the enclosure.

Background:

From the speed comparisons shown in Section 1.2.2, it should be obvious that running the Pi 500+ from the M.2 slot is much faster. And with a 1 TB NVMe SSD you also get more storage space!

Requirements:

Do the steps below, in the order shown, to complete the requirements for this example.

***Note*** The Raspberry Pi Foundation does not officially recommend or support upgrading an existing Bookworm image “in-place” to Trixie, especially for a complex or relied-upon system like your NVMe SSD setup.

0. The only officially supported method is to start fresh, or do a clean install. This method in general is-

a) Back up your data: Use dd or a tool like rsync to copy your important files and configurations from the current Bookworm system to some external medium.

b) Download Trixie: Get the latest Raspberry Pi OS (Trixie) 64-bit image from the official Raspberry Pi website, and save it to the original 256 GB NVMe SSD.

c) Flash the SSD: Use Raspberry Pi Imager, already available on the Raspberry Pi 500+, to write the Trixie image directly to your larger capacity M.2 SSD, using a USB3 NVMe adapter that contains the new, larger capacity NVMe SSD.

d) Reinstall and Restore: Shut the system down, and remove the original 256 GB NVMe SSD using instructions found online. The, install the new, larger NVMe SSD and close up the Pi 500+. Boot the Pi 500+ from the new Trixie SSD, do the initial system configuration, reinstall your applications, and restore your data from the backup external medium.

### 1. Flashing the new Trixie image -

***Note*** Since you're physically swapping the SSD into the internal slot after copying the OS image onto a new NVMe SSD, just make sure the Imager can see the new SSD through the adapter on your Raspberry Pi 500+.

a) Use the Raspberry Pi Imager - it has a built-in "Edit Settings" dialog (gear icon) where you can pre-configure hostname, username, Wi-Fi, SSH, and locale. This saves you from doing it on first boot, if you want to do that. We didn’t.

b) Since the Pi 500+ boots from the M.2 SSD, in our procedures here you'll need to use a USB-to-M.2 adapter (or an external NVMe enclosure) to write the image to the new NVMe SSD in that adapter mounted on the Pi 500+.

c) Flashing - Select the 64-bit Raspberry Pi OS (Trixie) image. Double-check you're writing to the 1 TB SSD and not accidentally to your 256 GB one or another drive. The Imager shows drive sizes, so verify the ~1 TB capacity before clicking Write in the Imager dialog box.

2. Switching the SSDs – After the Imager has copied the new OS image to the new NVMe SSD, shut down your Raspberry Pi 500+, and follow these online instructions on how to open the case and change the SSD:

https://www.raspberrypi.com/documentation/computers/keyboard-computers.html#replace-m2

### Your 500+ should have come with a handy tool to facilitate opening the case. When done, snap the case back together and replace the five screws.

### 3. First boot -

a) Reconnect necessary cables, and boot the Pi 500+ with the new larger capacity NVMe SSD in it. Give it a minute or two on first boot — it resizes the filesystem and does some initial setup.

b) First boot will take a bit longer (filesystem resize + initial setup). Be patient before assuming something's wrong. Once it's up: sudo apt update && sudo apt full-upgrade to pick up any post-release fixes.

c) Only then, when the system is running properly, start restoring your data from an external source (perhaps the original 256 GB NVMe SSD that came with the machine,) and install your favorite or required apps.

4. A safety net -

Keep your old Bookworm SSD untouched (don't wipe it) until the new Trixie install is fully working and you've confirmed everything you need is restored. That way you always have a fallback, to run the Pi 500+ from.

Keeping the 256 GB as a fallback

Label it clearly (e.g., a piece of tape marked "Bookworm — do not wipe") so  in the future you can use it- don't accidentally reuse it. If you ever want to boot from it again, it's just a swap back — there will be no reflashing needed.

***Note*** You can mount it occasionally via the USB adapter to pull any files you forgot to copy over, or to be the archive of your data that you might have put on that SSD.

***Note*** One small gotcha

If your Bookworm system used any hostname-based or machine-ID-based configs (some Docker setups, license keys, etc.), those will differ on the new install. Worth noting what you had before you swap, in case something doesn't "just work" after restore.

Conclusion: The clean install used to switch the original 256 GB NVMe SSD for a larger capacity storage medium is the only reliable and supported route for a system you depend on.

______________________________________________________________________________

1.9.1 Installing ZFS on the Raspberry Pi 5 OS based on Debian Trixie

Installing ZFS on the Raspberry Pi 5 OS at the time this book was written was a simple, straight forward operation. Use the following commands:

$ sudo apt update

Output truncated...

$ sudo apt install linux-headers-rpi-v8

Output truncated...

$  sudo apt install zfs-dkms zfsutils-linux

Output truncated...

After a few minutes, the software is installed.

If ZFS is loaded successfully, you can verify that as follows:

$  dmesg | grep ZFS

[ 2109.850475] ZFS: Loaded module v2.4.4-1~bpo13+0~rpt1, ZFS pool version 5000, ZFS filesystem version 5

You can also use the following commands to verify that ZFS has been installed correctly-

$ zfs - -version

zfs-2.4.4-1~bpo13+0~rpt1

zfs-kmod-2.4.4-1~bpo13+0~rpt1

$ zpool list

no pools available

$

______________________________________________________________________________

1.13 Software Updates and Operating System Upgrades

We begin this Section with two questions, which we try to answer for you to give you a better feel for why software updates and operating system updates are important.

Question: Why would the two most-significant, post-installation tasks, that an ordinary user on a single-user Raspberry Pi 5 system need to perform, be as follows?

a) The addition, removal, or update of software applications as packages on the system.

b) The upgrade of the operating system itself to a newer, currently stable release

Answer:

a) The standard installation of the operating system does not include software that the user needs, given the use-cases she puts the system to. And also, changing conditions in use-cases require removal of software packages as well. Finally, as newer and improved versions of installed packages become available, there must be a quick and easy way to update the packages to incorporate those improvements.

b) Purely from a performance and security viewpoint, upgrades of the Raspberry Pi 5 OS happen very regularly, and even an ordinary user should take advantage of these improvements via upgrades.

Question: What is the difference between “updating” and “ upgrading” a Raspberry Pi 5 system?

Answer: Generally in the literature, the two terms are often confused with one another. But from our perspective in this Section (and throughout this chapter), we define the two terms as follow:

Updating is the process of adding or replacing application software packages. For example, bringing the nginx web server application from an earlier-installed version to the latest stable release of it.

Upgrading is the process of replacing the entire operating system with a newer, hopefully stable release. The upgrade can be to a newer minor release, or to a new major release. For example, upgrading from what was named Raspbian to the Raspberry Pi OS, an upgrade which was not just a naming convention change. Additionally, a change in the kernel from an earlier release to a later release constitutes a major upgrade of the operating system.

In this Section we show the basics of how these two fundamental tasks can be done on our representative Raspberry Pi 5 OS.

For software application addition, removal, or updates, we limit the discussion to covering the Advanced Packaging Tool (APT), because that’s the default package manager on the Raspberry Pi 5 OS. APT can be used very effectively in a text-based, CUI environment. That’s how we approach it in this book. That doesn’t preclude use of GUI-based front ends to package managers, available via the Raspberry Pi menu Preferences > Add/Remove Software.

Additionally, we offer some information about Snaps here in this Section. The Snap package manager is a software package management system developed by Canonical, the company supporting Ubuntu Linux. It is designed to work across different Linux distributions, and provides a way to package and distribute software in a self-contained format called "Snaps."

Snaps are containerized software packages that include all the necessary dependencies and libraries required for the application to run, making them more isolated and secure. They aim to simplify software installation and updates by providing a consistent experience across different Linux distributions.

As for the Raspberry Pi 5 OS, the Snap package manager is not installed by default. However, it is possible to install Snap on Raspberry Pi 5 OS if desired. Canonical provides a version of the Snap package manager, that is compatible with ARM-based systems like the Raspberry Pi 5 OS. Instructions for installing Snap on Raspberry Pi 5 OS can be found on the official Snapcraft website, or through community resources.

Snap can be installed on a Raspberry Pi running the latest version of the Raspberry Pi 5 OS by opening a terminal, and typing the following commands.

$ sudo apt update

$ sudo apt install snapd

Output truncated…

$

You will need to then reboot your system.

$ sudo reboot

Then install the core snap so you can get the latest snapd.

$ sudo snap install core24

Output truncated...

core24 20260824 from Canonical✓ installed

$

***Note*** Modern snaps typically use newer bases like core22 or core24. These are installed automatically as dependencies when you install software that requires them, and they are generally the ones you want to keep updated. If you're specifically looking to install the latest base environment for modern applications, sudo snap refresh will handle those as needed.

For upgrading the operating system itself, we showed you a clean, fresh install in Section 1.2.5. and additionally a basic set of procedures in Section 1.13.4 that suits the variable conditions of upgrading an operating system.

In-Chapter Exercise

26. In the light of what we have said about updating and upgrading your Raspberry Pi 5 system, list applications that you want to install, given the use case(s) you undertake on your system. Then, as you go through the rest of this Section, use the APT package manager to install these.

The best way for you to get more basic help on the package management commands we show in this Section is to read the man pages available for them on your system.

______________________________________________________________________________

1.13.1.1 Default Repositories

On a Raspberry Pi 5 or 500+ running the latest Raspberry Pi 5 OS, APT sources are no longer listed in the traditional single file. They are now organized in separate files located in the /etc/apt/sources.list.d/ directory.

📂

Where to Find the Source Lists-

The main configuration directory is /etc/apt/sources.list.d/. The old /etc/apt/sources.list file is now typically empty or deprecated on modern installations.

You will usually find two key files here:

debian.sources: Points to the main Debian repositories (e.g., for core system packages).

raspi.sources: Points to the Raspberry Pi Foundation's repositories (for Raspberry Pi-specific software and updates).

How They Are Formatted-

These files use the newer DEB822 format, which is structured and easier to read than the old "One-Line-Style". Instead of cramming all information onto one line, it uses key-value pairs.

If you open one of these files, you will see fields like:

Types:: Usually deb for binary packages (or deb-src for source code).

URIs:: The actual web address of the repository.

Suites:: The specific release name (e.g., trixie or bookworm).

Components:: The sections to include (e.g., main, contrib).

Signed-By:: The path to the key used to verify the repository.

To open and edit debian.sources or raspi.sources, you need a text editor with sudo (root) permissions, since these files are owned by root and located in a system directory.

Common Commands

Using nano (easiest for beginners):

$ sudo nano /etc/apt/sources.list.d/debian.sources

Output truncated...

$

$ sudo nano /etc/apt/sources.list.d/raspi.sources

Output truncated…

***Note***

After editing either file, always run:

$ sudo apt update

This refreshes APT's package index so your changes take effect. Also, be careful not to change the Signed-By: paths or the Suites: names unless you know what you're doing — breaking these can prevent APT from verifying or finding packages.

For software that provides a system service, such as sshd, or a web server like nginx, the package manager not only downloads and installs all necessary components and their dependencies, but also uses systemd facilities to start the service, and ensure it automatically starts at every subsequent system boot.

If you just want to see which repositories APT is actually using (a summarized, parsed view), run:

$ apt policy

Output truncated…

This shows the configured repositories along with their priority and which packages are available from each. It's a great sanity check after viewing the raw files.

_____________________________________________________________________________

In Section 2.5.6, the sudo who -r commands, which are now deprecated, should be removed from the text. Add this command, and its output, and an additional paragraph at the end of that Section-

Then reexamine the current active targets with the following command:

$ systemctl list-units --type=target –state=active

truncated...

***Note*** The graphical.target or multi-user.target shown as active tells you the current state. Note that graphical.target actually pulls in multi-user.target as a dependency, so you'll often see both listed as active — the graphical one is the "highest" reached target and represents your current state.