Remote Printing from Your Pi 5 to Another Pi via WiFi

Objectives: To set up CUPS to print from LibreOffice Writer on a Raspberyy Pi 5 to a remote Raspberry Pi 400 via a WiFi connection.

Question: I have an printer attached via a USB cable to a "headless" Raspberry Pi 400 (running the Raspberry Pi “Bookworm” version of the OS) on my Wi-Fi network. How can I send documents to it from within LibreOffice Writer on my Raspberry Pi 5 (which has a full Desktop environment) to print them?

Procedure:

This is exactly what CUPS printer sharing is for.
Your Pi 400 will act as a network IPP printer.

Notes: On my LAN, the IP address of my Pi 400 is 192.168.1.72, and the IP address of my Pi 5 is 192.168.1.14. My printer on the Pi 400 is named HP-Laserjet-1200.

# STEP 1 — On the Pi 400 (the headless print server)

### a. Make sure CUPS is installed* and the HP printer is recognized

sudo apt update

sudo apt install cups hplip

* Do this on the Pi 5 as well, which, by default, has CUPS already installed when it’s running the Raspberry Pi “Trixie” version, so only install hplip.

### b. Add the printer admin privileges

sudo usermod -aG lpadmin $USER

### c. Enable printer sharing

Edit CUPS config:

sudo nano /etc/cups/cupsd.conf

Make sure all of these are present and NOT commented:

SharePrinters Yes

Browsing On

BrowseLocalProtocols dnssd

WebInterface Yes

Also update access blocks (important):

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

<Location /admin>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

<Ctrl-X> Save → Exit → Restart CUPS:

sudo systemctl restart cups

### d. Add the printer (if not already added)

From the Pi 5 in a web browser, launch  and interface with CUPS on the Pi 400 with the URL:

http://192.168.1.72:631

Add the HP LaserJet 1200 via:
Administration → Add Printer → HP LaserJet 1200 (USB)

Make sure to check:
✅ Share This Printer

# STEP 2 — Find the Printer’s Network Address

On the Pi 400, run:

hostname -I

Example result:

192.168.1.72

Now test from your Pi 5:

ping 192.168.1.72

# STEP 3 — On the Pi 5 (which is the client machine)

### a. Install  Avahi discovery

sudo apt install avahi-daemon

### b. Open LibreOffice Writer on an existing test file→ File → Print

If CUPS sharing is correct, the printer will automatically appear as:

HP LaserJet 1200 @ raspberrypi400

If it appears → you’re done. ✅

# If it does not appear automatically, add it manually

Go to the Pi 5 CUPS web interface:

http://localhost:631/admin

Choose:
Add Printer → Enter URI

Enter the following (to replace the IP with your Pi 400’s IP, or the IP of your Pi print server):

ipp://192.168.1.72/printers/HP_LaserJet_1200

Or browse:
Internet Printing Protocol (IPP) → your printer → Add.

Select a driver:

- HP LaserJet 1200 Foomatic/PostScript

- or just Generic PCL 5e

Save.

Now Print test file.

# Done. Printing should now work from LibreOffice on the Pi 5 and all apps. 🎉

## If you want to verify everything, use these commands on the Pi 400:

### 1. The Pi 400’s IP:

hostname -I

### 2. The printer name from:

lpstat -p -d

But what if:

In sub-step c. of Step 1 the Pi 400 CUPS will not restart.

If CUPS will not restart after editing /etc/cups/cupsd.conf, the config file syntax is likely invalid.
CUPS is strict — one misplaced space or brace will cause failure.

Let’s verify and fix quickly.

## 1. Check why CUPS failed

Run this:

sudo journalctl -u cups -n 50 --no-pager

You will likely see a message like:

Syntax error on line XX of /etc/cups/cupsd.conf

## 2. Validate the config directly

This command checks the syntax without restarting:

sudo cupsd -t

If it prints:

Syntax OK

then we can start safely:

sudo systemctl restart cups

If it prints an error, note what it is exactly as appears on screen.

## 3. The Most Common Cause

Usually the error happens if this block was typed incorrectly:

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

CUPS requires the Order allow,deny line to come before the Allow lines — otherwise it fails.

It must be exactly:

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

Same structure in /admin:

<Location /admin>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

## 4. Quick Fix: Restore a Known-Good Minimal Working Config

If you want to repair immediately without chasing syntax:

sudo cp /etc/cups/cupsd.conf /etc/cups/cupsd.conf.backup

sudo bash -c 'cat >/etc/cups/cupsd.conf' <<'EOF'

LogLevel warn

WebInterface Yes

DefaultAuthType Basic

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

<Location /admin>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

Port 631

Listen /run/cups/cups.sock

Browsing On

BrowseLocalProtocols dnssd

SharePrinters Yes

EOF

Then:

sudo systemctl restart cups

## 5. Finalizing the working config

What is your network range?

Run on either Pi:

hostname -I

You’ll get something like:

| Example IP | Correct Allow line |
| --- | --- |
| 192.168.1.44 | Allow 192.168.1.0/24 |
| 192.168.0.22 | Allow 192.168.0.0/24 |
| 10.0.0.57 | Allow 10.0.0.0/24 |

Once you know the IP you will produce a final exact cupsd.conf for your network (no guessing, no editing).

### Reply with:

- Output of:

hostname -I

- Output of:

sudo cupsd -t

If the IP address of the Pi400 is 192.168.1.72, for example.

Since your Pi 400 is 192.168.1.72, here’s a known-good CUPS config for that subnet that should restart cleanly and allow sharing to your LAN.

## 1) Drop in a minimal, valid cupsd.conf

# backup first

sudo cp /etc/cups/cupsd.conf /etc/cups/cupsd.conf.backup.$(date +%F-%H%M)

# write a clean config

sudo bash -c 'cat > /etc/cups/cupsd.conf' << "EOF"

LogLevel warn

WebInterface Yes

DefaultAuthType Basic

# Listen on both IPv4/IPv6 and the local socket

Port 631

Listen /run/cups/cups.sock

# Advertise shared printers on the LAN

Browsing On

BrowseLocalProtocols dnssd

SharePrinters Yes

DefaultShared Yes

# Allow the whole local subnet (192.168.1.0/24) plus localhost

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

# Admin pages (you'll still be prompted for a user in lpadmin group)

<Location /admin>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

AuthType Default

Require user @SYSTEM

</Location>

# Editing config via the web UI

<Location /admin/conf>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

AuthType Default

Require user @SYSTEM

</Location>

EOF

## 2) Validate and restart CUPS

sudo cupsd -t            # should print: Syntax OK

sudo systemctl restart cups

sudo systemctl status cups --no-pager

If cupsd -t doesn’t say “Syntax OK”, go back and fix the syntax!

## 3) (If you use UFW) open the port to your LAN

sudo ufw allow from 192.168.1.0/24 to any port 631 proto tcp

## 4) Make sure the printer is added & shared on the Pi 400

If you haven’t added it yet:

- From the Pi 5 in a web browser URL type http://192.168.1.72:631 → Administration → Add Printer → HP LaserJet 1200 (USB)

- Check “Share This Printer.”

Confirm it’s there and shared:

lpstat -p -d

lpoptions -p HP_LaserJet_1200 -l | head

## 5) From your Pi 5 (LibreOffice Writer)

You should now see the printer automatically (thanks to DNSSD). If it doesn’t pop up immediately, add it once via CUPS on the Pi 5:

- Go to http://localhost:631/admin → Add Printer → Enter URI and paste:

ipp://192.168.1.72/printers/HP_LaserJet_1200

- Driver: HP LaserJet 1200 Foomatic/PostScript (or Generic PCL 5e if that’s all you see).

- Save → Print Test Page.

### Quick checks

From the Pi 5:

curl -I http://192.168.1.72:631         # expect HTTP/1.1 200 OK or 401/403 (means it’s reachable)

HTTP/1.1 200 OK

Connection: Keep-Alive

Content-Language: en_US

Content-Length: 2262

Content-Type: text/html; charset=utf-8

Date: Sat, 08 Nov 2025 16:38:26 GMT

Keep-Alive: timeout=10

Last-Modified: Sun, 07 Sep 2025 17:45:05 GMT

Accept-Encoding: gzip, deflate, identity

Server: CUPS/2.4 IPP/2.1

X-Frame-Options: DENY

Content-Security-Policy: frame-ancestors 'none'

lpstat -e                               # should list the shared printer

Hewlett_Packard_HP_LaserJet_1200_raspberrypi

That’s it—LibreOffice Writer will use that printer like any other. If anything still refuses to start, run:

sudo journalctl -u cups -n 80 --no-pager

and paste the last error line; that’ll pinpoint the fix.

If sudo cupsd -t has a problem with SharePrinters: then:

On modern CUPS (incl. Raspberry Pi OS/Debian Bookworm/Trixie), SharePrinters is not a valid directive, so cupsd -t will fail.

Use these instead:

Browsing On

BrowseLocalProtocols dnssd

DefaultShared Yes ← enables sharing by default

And make sure each printer itself is marked shared (printer-is-shared=true)

## Quick fix

- Edit the config and remove the bad line:

sudo nano /etc/cups/cupsd.conf

Use this minimal, known-good set (replace the whole file if you like):

LogLevel warn

WebInterface Yes

DefaultAuthType Basic

Port 631

Listen /run/cups/cups.sock

Browsing On

BrowseLocalProtocols dnssd

DefaultShared Yes

<Location />

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

</Location>

<Location /admin>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

AuthType Default

Require user @SYSTEM

</Location>

<Location /admin/conf>

Order allow,deny

Allow localhost

Allow 192.168.1.0/24

AuthType Default

Require user @SYSTEM

</Location>

- Validate and restart:

sudo cupsd -t         # should say: Syntax OK

sudo systemctl restart cups

- Ensure the printer itself is shared:

# replace with your actual CUPS name, e.g. HP_LaserJet_1200

lpstat -p -d

sudo lpadmin -p HP_LaserJet_1200 -o printer-is-shared=true

- (Recommended) Make sure Avahi is broadcasting the service:

sudo apt install avahi-daemon

sudo systemctl enable avahi-daemon --now

- From your Pi 5, it should now appear automatically in LibreOffice’s printer list.
If not, add it once via URI:

ipp://192.168.1.72/printers/HP_LaserJet_1200

It should work now.

That means your Pi 400 is now officially acting as a network print server, and your Pi 5 (and anything else on your LAN) can print to the HP LaserJet 1200 cleanly.

You now have:

- ✅ CUPS configured correctly

- ✅ Printer sharing enabled

- ✅ Network printing working from LibreOffice

- ✅ Old but wonderful laser printer given a new life

This is a very solid setup.